ENGR480	LinuxCNC Exercise						2017-11-15

We will be controlling two linear motion axes using LinuxCNC today.

Parts needed:

(1) windows laptop

(1) Optiplex 790 box

(2) AutomationDirect STP-DRV-6575 stepper drives

(2) SLA-90 linear actuators

(2) four wire cables

(1) 48V power supply (from your PLC kit)

(1) 25 pin ribbon cable

(1) 25 pin breakout

Setup:

1. On the STP-DRV-6575's, set the motor switch to position 8 (2amps per phase, STP-MTR-17048 motor).

2. On the STP-DRV-6575's, set SW5,6,7 to on-on-off (2000 steps/rev), SW1,2,3,4=off-off-on-on

3. Wire one 4-wire cable to the motor connector: A+=BLACK, A-=GREEN,B+=WHITE, B-=RED

4. Wire the parallel port breakout to the stepper drives:

Parallel Port Pin	Signal			X Amplifier	Y Amplifier

2		X Step			+Step

3		X Direction		+Dir

4		Y Step					+Step

5		Y Direction				+Dir

18		ground			-Step

19		ground			-Dir

20		ground					-Step

21		ground					-Dir

5. Wire your 48v to the V- and V+ terminals on the motor connectors and to the POWER IN - and + on the Machinekit unit.

6. Plug the motor cables into the SLA90 actuators (these cables have the 4-pin male-male connectors)

8. Go to Win10 start menu, click on Xlaunch.

9. Go to Win10 start menu, click on Putty.

10. Type the IP address on the sticky note for your 790 in the address box, then click on SSH -> X11, and check the "X11 Forwarding" box.  Go back to Session, type "linuxcnc" in configuration box, then "save".  Now click "open".

11. Once you have putty running, it should connect to the linuxcnc machine, and ask for login and password, which is "mfg" and "krh105".

12. In your putty (linuxcnc) window, type:

cd linuxcnc/configs/engr480

stepconf

13. In stepconf, use this information:

Base Information: Machine Name: engr480, Axis config: XYZ, Machine units: MM

Parallel Port 1: leave defaults

Options: uncheck onscreen prompt for tool change

Axis X: 200 motor steps/rev, 10 microstepping, 10mm/rev leadscrew pitch

Axis Y: 200, 10, 10

14. After leaving stepconf, type 'linuxcnc', and choose the engr480 configuration you just made.

15. Now that LinuxCNC is running, you can run your actuators.  To do this, follow these steps:

a) Click on the (X) and (I) icons to turn the controller on and remove E-stop.

b) In Manual control tab, home each axis

c) In MDI tab, type "G1 X10.0 Y10.0 F5000" to move each axis 10mm.  If no motion is observed, figure out why 	before proceeding.

16. Open Notepad on the PC to create a simple CNC program:

G1 X0.0 Y0.0 F5000.0

G1 X20.0 Y30.0

G1 X10.0 Y10.0

G2 X0.0 Y0.0 R10.0

17. Save your file, close Notepad and copy your file to the linuxcnc computer by typing:

d:\pscp  whateveryoucalledit.txt  mfg@youripaddr:linuxcnc/nc_files/whateveryoucalledit.ngc

18. In LinuxCNC, load your program with File->Open, then press the ► button.

19. When done, exit LinuxCNC, they type: "sudo shutdown -h now" for a clean shutdown.

Documentation is at https://linuxcnc.org.